# Firewall Traffic Flow Tables - Outside to Inside

Palo Alto NGFW and Cisco Firepower Threat Defense (FTD)

| Organization | [Company name] | Document ID | NET-01 |
| --- | --- | --- | --- |
| Version | 1.0 | Effective date | [Date] |
| Policy owner | Network Security Lead | Approved by | [Name, Title] |
| Review frequency | At least annually, and after any significant incident or change | Classification | Internal |

## 1. Purpose

This document lists every network flow allowed from the internet (outside) to internal servers, with the public and internal addresses, the application and transport protocol, and the firewall rule and NAT rule that implement it, on both the Palo Alto and the Cisco Firepower firewall. All other inbound traffic is denied by default and logged.

## 2. Scope

Covers inbound traffic from the internet / core to the Palo Alto and Cisco FTD inside networks for the scored and business-required services, plus the internal flows that cross both firewalls. Outbound and inter-zone flows are covered by the Network Security Policy (POL-08).

## 3. Definitions

| Term | Definition |
| --- | --- |
| NAT | Network Address Translation; here, static one-to-one mapping between a public 172.25.(20+N).x address and an internal 172.20.x.x server. |
| Pre-NAT / post-NAT | The packet's addresses before and after translation. |
| App-ID | Palo Alto identification of the application in the traffic, independent of port. |
| application-default | Palo Alto service setting that allows an application only on its standard ports. |
| IP protocol number | IANA number in the IP header for the transport protocol: ICMP = 1, TCP = 6, UDP = 17. |
| Auto NAT | Firepower NAT defined on a network object. |

## 4. Zones and Addressing

| Segment | Device / interface | Network | Hosts (2026 MWCCDC topology) |
| --- | --- | --- | --- |
| Core / internet | VyOS router external | 172.31.(20+N).2/29 to core 172.31.(20+N).1; public pool 172.25.(20+N).0/24 | Scoring engine, red team, internet (N = team number) |
| Router net1 | VyOS 172.16.101.1 / Palo Alto outside 172.16.101.254 | 172.16.101.0/24 | Transit to the Palo Alto |
| Router net2 | VyOS 172.16.102.1 / FTD outside 172.16.102.254 | 172.16.102.0/24 | Transit to the Cisco FTD |
| Palo Alto inside | Palo Alto inside 172.20.242.254 (mgmt 172.20.242.150) | 172.20.242.0/24 | E-com 172.20.242.30, Webmail 172.20.242.40, Splunk 172.20.242.20, Ubuntu workstation (DHCP) |
| FTD inside | FTD inside 172.20.240.254 (mgmt 172.20.240.200) | 172.20.240.0/24 | AD/DNS 172.20.240.102, Web 172.20.240.101, FTP 172.20.240.104, Windows 11 172.20.240.100 |

## 5. Outside-to-Inside Flows - Palo Alto

| # | Service | Destination - public (pre-NAT) IP | Internal host (post-NAT) | Destination zone (post-NAT) | App-ID | Transport protocol (IP proto #) / port | PA security rule | PA NAT rule |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | E-commerce HTTP / HTTPS | 172.25.(20+N).11 | 172.20.242.30 (Ubuntu E-com) | inside | web-browsing, ssl | TCP (6) / 80, 443 | IN-ECOM-WEB | Static NAT 172.20.242.30 <-> .11 (bi-directional) |
| 2 | Webmail SMTP | 172.25.(20+N).39 | 172.20.242.40 (Fedora webmail) | inside | smtp | TCP (6) / 25 | IN-MAIL | Static NAT 172.20.242.40 <-> .39 |
| 3 | Webmail POP3 | 172.25.(20+N).39 | 172.20.242.40 | inside | pop3 | TCP (6) / 110 | IN-MAIL | same |
| 4 | Webmail web UI (if scored) | 172.25.(20+N).39 | 172.20.242.40 | inside | web-browsing, ssl | TCP (6) / 80, 443 | IN-MAIL-WEB | same |
| 5 | Splunk web (only if required) | 172.25.(20+N).9 | 172.20.242.20 (Splunk) | inside | web-browsing | TCP (6) / 8000 | IN-SPLUNK-WEB (disabled unless required) | Static NAT 172.20.242.20 <-> .9 |
| 6 | Ping to every host (competition rule) | 172.25.(20+N).11, .39, .9 | Hosts in rows 1-5 | inside | ping | ICMP (1) / type 8 echo-request | IN-PING | As above |
| 7 | Everything else | any | any | any | any | any | IN-DENY-LOG (deny, log at session end) | - |

- N = team number: public addresses are 172.25.(20+N).x, e.g. team 5 = 172.25.25.x (2026 MWCCDC team packet). The scoring portal lists the exact scored services - check each row against it.

- Palo Alto matches security rules on the pre-NAT destination IP and the post-NAT destination zone. So the rule's destination address is the public 172.25.(20+N).x address and the destination zone is the inside zone where the server lives.

- Each public server has one static bi-directional NAT rule, so one rule handles inbound and outbound translation.

- Service is application-default on every allow rule, so an application is only allowed on its standard port.

- Rules are evaluated top-down, first match wins: rows 1-6 must sit above any broad allow rule. Older saved configurations contain broad rules (any source, destination and application, allow) above the specific rules, which makes every rule below them ineffective. Remove or disable such rules once rows 1-7 are in place and tested.

- Every allow and deny rule logs at session end, and logs are forwarded to Splunk (syslog), so denied scored traffic is visible immediately.

- The Ubuntu workstation has no public address and no inbound rule. Firewall management (172.20.242.150) is only reachable from the inside.

- Competition rule: ICMP must stay up on every device except the Palo Alto core port, so row 6 is required.

## 6. Outside-to-Inside Flows - Cisco Firepower (FTD)

| # | Access control rule | Source zone | Destination zone | Destination network (real / internal IP) | Transport protocol (IP proto #) / port | Application | Action | NAT (auto-NAT, static, inside -> outside) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Allow-DNS | outside | inside | obj-ad-dns 172.20.240.102 | UDP (17) / 53; TCP (6) / 53 | DNS | Allow + intrusion policy | obj-ad-dns: static 172.20.240.102 -> 172.25.(20+N).155 |
| 2 | Allow-Web | outside | inside | obj-web 172.20.240.101 | TCP (6) / 80, 443 | HTTP, HTTPS | Allow + intrusion policy | obj-web: static 172.20.240.101 -> 172.25.(20+N).140 |
| 3 | Allow-FTP | outside | inside | obj-ftp 172.20.240.104 | TCP (6) / 21 (data channels opened by FTP inspection) | FTP | Allow + intrusion policy | obj-ftp: static 172.20.240.104 -> 172.25.(20+N).162 |
| 4 | Allow-Ping | outside | inside | All four hosts (real IPs, incl. Windows 11 172.20.240.100) | ICMP (1) / type 8 | ICMP | Allow | obj-win11: static 172.20.240.100 -> 172.25.(20+N).144 (+ rows 1-3) |
| Default | Default action | any | any | any | any | any | Block all traffic, log at connection start | - |

- Unlike Palo Alto, Firepower (FTD) access control rules match the real (internal, untranslated) IP address, so the destination network is the server's 172.20.240.x object, not the public address.

- NAT is configured as an Auto NAT rule on each network object: static, from the inside interface to the outside interface, translated to the public address (e.g. object obj-web 172.20.240.101, static to 172.25.(20+N).140).

- FTP: keep FTP inspection enabled (it is on by default) so the FTD opens the passive data ports automatically; otherwise the FTP server's passive port range must be allowed too.

- Access rules are evaluated top-down; the default action is Block all traffic with logging, which implements deny-by-default.

- Allow rules use an intrusion policy (e.g. Balanced Security and Connectivity) and log at the end of the connection, sent to Splunk via syslog.

- Changes take effect only after Deploy (FDM: Deploy Changes). FTD management: 172.20.240.200, inside only.

## 7. Internal Flows Crossing Both Firewalls

The two halves of the network reach each other through the VyOS router, so these internal flows must be allowed on both firewalls (and by the host firewalls):

| Flow | Source | Destination | Protocol / port | Allow on |
| --- | --- | --- | --- | --- |
| Webmail logins checked against AD (POP3 users are AD accounts) | 172.20.242.40 | 172.20.240.102 | TCP 389/636 LDAP(S), TCP+UDP 88 Kerberos | Palo Alto (outbound) and FTD (inbound) |
| DNS and NTP for the Palo Alto-side Linux boxes | 172.20.242.0/24 | 172.20.240.102 | UDP+TCP 53, UDP 123 | Palo Alto and FTD |
| Log forwarding to Splunk | 172.20.240.0/24 (Windows) | 172.20.242.20 | TCP 9997 (forwarder), UDP 514 (syslog) | FTD (outbound) and Palo Alto (inbound) |
| Firewall logs to Splunk | 172.20.242.150 (PA), 172.20.240.200 (FTD) | 172.20.242.20 | UDP 514 syslog | FTD side: FTD and Palo Alto |
| Admin from the Ubuntu workstation | Ubuntu workstation (DHCP, 172.20.242.x) | Linux boxes / Splunk / Palo Alto mgmt | TCP 22, 8000, 443 | Palo Alto (same side: host firewalls only) |

## 8. Verification

Prove each flow after every change:

- Palo Alto (CLI): test security-policy-match from outside to inside source 198.51.100.10 destination 172.25.(20+N).11 destination-port 80 protocol 6 (use your zone names) - expect rule IN-ECOM-WEB. Monitor > Traffic shows real sessions and their matched rule.

- Firepower (CLI): packet-tracer input outside tcp 198.51.100.10 40000 172.25.(20+N).140 80 - expect the NAT translation and rule Allow-Web. Monitoring > Events shows real connections.

- From outside, test each scored service (web pages load over HTTP and HTTPS, SMTP banner on 25, POP3 login, DNS query answers, FTP login), check the scoring portal, and confirm denied traffic appears in the logs for anything else. linux/portscan.sh shows which ports are reachable on every host.

## 9. Requirements Implemented

9.1 Inbound traffic is denied by default and only documented flows are allowed. [800-53: SC-7, SC-7(5), AC-4 | 27002: 8.20, 8.22 | CIS: 4.4, 13.4]

9.2 Each allowed flow is limited to the required destination host, application and port. [800-53: CM-7, AC-4 | 27002: 8.21 | CIS: 4.8, 12.2]

9.3 Firewall rule decisions are logged and sent to central log management. [800-53: AU-2, AU-12, SI-4 | 27002: 8.15, 8.16 | CIS: 8.2, 13.6]

9.4 This flow documentation is kept current and reviewed after each firewall change. [800-53: CM-3, PL-2 | 27002: 8.32 | CIS: 12.4]

## 10. Control Mapping

Controls each section implements. NIST = NIST SP 800-53 Rev. 5; ISO = ISO/IEC 27002:2022; CIS = CIS Critical Security Controls v8.1 safeguards.

| Section | NIST SP 800-53 Rev. 5 | ISO/IEC 27002:2022 | CIS Controls v8.1 |
| --- | --- | --- | --- |
| Requirements Implemented | AC-4, AU-2, AU-12, CM-3, CM-7, PL-2, SC-7, SC-7(5), SI-4 | 8.15, 8.16, 8.20, 8.21, 8.22, 8.32 | 4.4, 4.8, 8.2, 12.2, 12.4, 13.4, 13.6 |

## 11. Sources

- NIST SP 800-53 Rev. 5, Security and Privacy Controls for Information Systems and Organizations

- ISO/IEC 27002:2022, Information security, cybersecurity and privacy protection - Information security controls

- Center for Internet Security (CIS) Critical Security Controls v8.1

- NIST SP 800-41 Rev. 1, Guidelines on Firewalls and Firewall Policy

- IANA Protocol Numbers registry

- Palo Alto Networks PAN-OS Administrator's Guide - Policy, NAT

- Cisco Secure Firewall Threat Defense (FDM/FMC) Configuration Guide - Access Control, NAT

## 12. Revision History

| Version | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | [Date] | [Team #] | Initial release |